Wireshark Exploration

IT&C 101 - Intro to Cybersecurity

Purpose

As an IT&C student, networks are fundamental to every cyber system. Networks are a system of connected devices that allows us to share information. We send data to each other in packets, which are segments of data broken up from an original message to be sent across networks. Many different jobs are focused on building, maintaining, and fixing networks. Without networks, we would not have the Internet and our modern world. So, it’s a good idea to start learning the basics! Many different attacks and hacks can focus on the network and disrupt many different types of systems. Data is sent on networks through packets, and the tool to investigate and record packets is called Wireshark. This homework is to learn about the basics of Wireshark to create packet capture files and look at their contents.

Download

Link to download Wireshark: https://www.wireshark.org/download.html

Make sure to download the correct file for your computer architecture.

Windows: Download x64 installer

Mac: If you have an M1 or M2 chip, download ARM version. Otherwise download the Intel version.

For the installer, choose the default settings.

Objective

The objective of this lab is to create a pcap (packet capture) file and investigate its contents to see what Wireshark is doing, while gaining a deeper understanding of networks and network protocols.

DISCLAIMER: Only use this tool ETHICALLY. You can get into trouble for using Wireshark for nefarious purposes or using it to record network traffic for non-consenting parties.

Procedure

Packet Capture

Open Wireshark. You should see a screen that looks like this.

Double click on Wi-Fi to start the packet capture of the Wi-Fi network.

It might take a second for the packet capture to start.

When the packets start populating the page, go to your browser and navigate to byu.edu. After the page loads, return to Wireshark and stop the packet capture.

The stop button is in the top left corner of the page.

You have successfully recorded network traffic! Let’s investigate the packets.

Investigation

Follow along and answer the questions that appear.

Click on a packet. You will see a screen like this:

The anatomy of each line is:

Number: The id of the packets recorded in ascending order

Time: Time since start of recording

Source: Source IP address

Destination: Destination IP Address

Protocol: The Protocol the packet is “using”

Length: How much data is in the packet / how long it is

Info: What the packet contains; the data

The bottom left panel is the Packet Details Pane. It will tell you about the protocols of the packet, source and destination port, time of the packet and so much more.

The bottom right panel is the Packet Bytes Pane and displays the raw bytes / content of the packet.

Question 1: What is some other info you can learn from the Packet Details Pane?

Question 2: Scroll to the bottom. How many packets did you capture? In what amount of time? Did this surprise you? Why or why not?

There are a lot of packets, most likely. So, let’s learn about ways to filter packets.

There is an Apply Filter bar near the top of the page:

You can type a protocol into the filter and only packets of that protocol will appear.

Press the X in the top right corner of the filter bar to remove the filter.

You can also search packets for specific data. If you press Ctrl + F on Windows or Command + F on Mac (or Edit -> find Packet), more specific options will appear on the Filter Bar.

You can specify the display filter to Hex Value, String or Regular Expression. It will search for the packets that contain that data. Click “Find” to find the first packet with this data, then click it again to go to the next packet with that data.

Question 3: Search “byu.edu” as a string and find the packet that contains Client Hello (SNI = byu.edu) data. What is the destination IP address of that packet?

Question 4: Type the destination IP address into your browser’s address bar. Where does it take you? Why?

Streams

Data will be separated into different packets based on what the data is but still needs to go to the same place. A stream allows us to see the contents of all the packets headed to a certain place at a certain time.

In Wireshark, click Analyze (at the top of the screen) -> Follow -> TCP Stream. You will see the contents of all the packets associated with one stream. You should see a screen like this:

This is all the contents of the packets in a stream in one place. The packets are heading to the same places, at very similar times.

Question 5: Why can’t you read too much, if anything, from the contents of a TCP stream or the individual packets? What is TLS?

Submission

Turn in this file with your answers to Learning Suite.